# F' Dictionary - Bảng Tổng Hợp Chi Tiết (Bản Đầy Đủ)

Tài liệu này liệt kê chi tiết bao gồm mọi trường thông tin con (items) bên trong các cấu trúc dữ liệu, lệnh, sự kiện và kênh đo lường.

# 1. Kiểu dữ liệu (Type Definitions) - 35 mục

| Tên (Name) | Loại (Kind) | Chi tiết (Items/Members) | Mô tả (Annotation) |
| --- | --- | --- | --- |
| FwPrmIdType | Alias | Kiểu gốc: U32 | The type of a parameter identifier |
| Vspc.DataHandling.PrmWriteError | Enum | Các giá trị liệt kê (Enum): • OPEN: 0 • DELIMITER: 1 • DELIMITER_SIZE: 2 • RECORD_SIZE: 3 • RECORD_SIZE_SIZE: 4 • PARAMETER_ID: 5 • PARAMETER_ID_SIZE: 6 • PARAMETER_VALUE: 7 • PARAMETER_VALUE_SIZE: 8 | Parameter write error |
| Svc.ComQueueDepth | Array | Mảng 2 phần tử kiểu U32 | Array of queue depths for Fw::Com types |
| Svc.VersionType | Enum | Các giá trị liệt kê (Enum): • PROJECT: 0 - project version • FRAMEWORK: 1 - framework version • LIBRARY: 2 - library version • CUSTOM: 3 - custom version • ALL: 4 - all above versions | An enumeration for Version Type |
| Svc.BuffQueueDepth | Array | Mảng 1 phần tử kiểu U32 | Array of queue depths for Fw::Buffer types |
| Svc.QueueType | Enum | Các giá trị liệt kê (Enum): • COM_QUEUE: 0 • BUFFER_QUEUE: 1 | An enumeration of queue data types |
| Fw.DpState | Enum | Các giá trị liệt kê (Enum): • UNTRANSMITTED: 0 - The untransmitted state • PARTIAL: 1 - The partially transmitted state A data product is in this state from the start of transmission until transmission is complete. • TRANSMITTED: 2 - The transmitted state |  |
| FwDpPriorityType | Alias | Kiểu gốc: U32 | The type of a data product priority |
| Svc.EventManager.FilterSeverity | Enum | Các giá trị liệt kê (Enum): • WARNING_HI: 0 - Filter WARNING_HI events • WARNING_LO: 1 - Filter WARNING_LO events • COMMAND: 2 - Filter COMMAND events • ACTIVITY_HI: 3 - Filter ACTIVITY_HI events • ACTIVITY_LO: 4 - Filter ACTIVITY_LO events • DIAGNOSTIC: 5 - Filter DIAGNOSTIC events | Severity level for event filtering Similar to Fw::LogSeverity, but no FATAL event |
| FwChanIdType | Alias | Kiểu gốc: U32 | The type of a telemetry channel identifier |
| Svc.FprimeRouter.AllocationReason | Enum | Các giá trị liệt kê (Enum): • FILE_UPLINK: 0 - Buffer allocation for file uplink • USER_BUFFER: 1 - Buffer allocation for user handled buffer |  |
| FwDpIdType | Alias | Kiểu gốc: U32 | The type of a data product identifier |
| Fw.CmdResponse | Enum | Các giá trị liệt kê (Enum): • OK: 0 - Command successfully executed • INVALID_OPCODE: 1 - Invalid opcode dispatched • VALIDATION_ERROR: 2 - Command failed validation • FORMAT_ERROR: 3 - Command failed to deserialize • EXECUTION_ERROR: 4 - Command had execution error • BUSY: 5 - Component busy | Enum representing a command response |
| FwIdType | Alias | Kiểu gốc: U32 | The id type. |
| Svc.EventManager.Enabled | Enum | Các giá trị liệt kê (Enum): • ENABLED: 0 - Enabled state • DISABLED: 1 - Disabled state | Enabled and disabled state |
| Fw.DpCfg.ProcType | Enum | Các giá trị liệt kê (Enum): • PROC_TYPE_ZERO: 1 - Processing type 0 • PROC_TYPE_ONE: 2 - Processing type 1 • PROC_TYPE_TWO: 4 - Processing type 2 | A bit mask for selecting the type of processing to perform on a container before writing it to disk. |
| FwTlmPacketizeIdType | Alias | Kiểu gốc: U16 | The type of a telemetry packet identifier |
| TimeBase | Enum | Các giá trị liệt kê (Enum): • TB_NONE: 0 - No time base has been established (Required) • TB_PROC_TIME: 1 - Indicates time is processor cycle time. Not tied to external time • TB_WORKSTATION_TIME: 2 - Time as reported on workstation where software is running. For testing. (Required) • TB_SC_TIME: 3 - Time as reported by the spacecraft clock. • TB_DONT_CARE: 65535 - Don't care value for sequences. If FwTimeBaseStoreType is changed, value should be changed (Required) | Define enumeration for Time base types |
| FwPacketDescriptorType | Alias | Kiểu gốc: U16 | The width of packet descriptors when they are serialized by the framework |
| ComCfg.Apid | Enum | Các giá trị liệt kê (Enum): • FW_PACKET_COMMAND: 0 - Command packet type - incoming • FW_PACKET_TELEM: 1 - Telemetry packet type - outgoing • FW_PACKET_LOG: 2 - Log type - outgoing • FW_PACKET_FILE: 3 - File type - incoming and outgoing • FW_PACKET_PACKETIZED_TLM: 4 - Packetized telemetry packet type • FW_PACKET_DP: 5 - Data Product packet type • FW_PACKET_IDLE: 6 - F Prime idle • FW_PACKET_HAND: 254 - F Prime handshake • FW_PACKET_UNKNOWN: 255 - F Prime unknown packet • SPP_IDLE_PACKET: 2047 - Per Space Packet Standard, all 1s (11bits) is reserved for Idle Packets • INVALID_UNINITIALIZED: 2048 - Anything equal or higher value is invalid and should not be used | APIDs are 11 bits in the Space Packet protocol, so we use U16. Max value 7FF |
| FwTimeContextStoreType | Alias | Kiểu gốc: U8 | The type used to serialize a time context value |
| Svc.VersionEnabled | Enum | Các giá trị liệt kê (Enum): • DISABLED: 0 - verbosity disabled • ENABLED: 1 - verbosity enabled | Tracks versions for project, framework and user defined versions etc |
| Svc.VersionCfg.VersionEnum | Enum | Các giá trị liệt kê (Enum): • PROJECT_VERSION_00: 0 - Entry 0 | Define a set of Version entries on a project-specific basis. |
| FwSizeType | Alias | Kiểu gốc: U32 | The unsigned type of larger sizes internal to the software, e.g., memory buffer sizes, file sizes. Must be unsigned. |
| FwOpcodeType | Alias | Kiểu gốc: U32 | The type of a command opcode |
| ISIS.FramStatus | Enum | Các giá trị liệt kê (Enum): • OK: 0 • START_SPI_FAIL: 1 - FRAM_start failed because SPI driver init failed • START_LOCK_CREATE_FAIL: 2 - FRAM_start failed because semaphore/lock creation failed • LOCK_FAIL: 3 - FRAM operation failed because FRAM access lock could not be obtained • INVALID_ARGUMENT: 4 - Invalid input, null argument, out-of-range address, or invalid size • TRANSFER_FAIL: 5 - FRAM write or SPI transfer failed • VERIFY_FAIL: 6 - FRAM writeAndVerify failed because read-back data mismatched • NOT_STARTED: 7 - FRAM_start has not been called successfully • NULL_BUFFER: 8 - Fw.Buffer data pointer is null • SIZE_TOO_LARGE: 9 - Requested size is larger than Fw.Buffer size • UNKNOWN_ERROR: 255 - Unexpected raw return code from low-level FRAM driver |  |
| Vspc.DataHandling.PrmReadError | Enum | Các giá trị liệt kê (Enum): • OPEN: 0 • DELIMITER: 1 • DELIMITER_SIZE: 2 • DELIMITER_VALUE: 3 • RECORD_SIZE: 4 • RECORD_SIZE_SIZE: 5 • RECORD_SIZE_VALUE: 6 • PARAMETER_ID: 7 • PARAMETER_ID_SIZE: 8 • PARAMETER_VALUE: 9 • PARAMETER_VALUE_SIZE: 10 | Parameter read error |
| FwSizeStoreType | Alias | Kiểu gốc: U16 | The type used to serialize a size value |
| Svc.VersionStatus | Enum | Các giá trị liệt kê (Enum): • OK: 0 - Version was good • FAILURE: 1 - Failure to get version | An enumeration for version status |
| PlatformSizeType | Alias | Kiểu gốc: U32 | The unsigned type of larger sizes internal to the software, e.g., memory buffer sizes, file sizes. Must be unsigned. Supplied by platform, overridable by project. |
| Fw.DeserialStatus | Enum | Các giá trị liệt kê (Enum): • OK: 0 • BUFFER_EMPTY: 3 - Deserialization buffer was empty when trying to read data • FORMAT_ERROR: 4 - Deserialization data had incorrect values (unexpected data types) • SIZE_MISMATCH: 5 - Data was left in in the buffer, but not enough to deserialize • TYPE_MISMATCH: 6 - Deserialized type ID didn't match | Deserialization status |
| Svc.CustomVersionDb | Struct | Các thành phần (Members): • version_enum (Svc.VersionCfg.VersionEnum) - enumeration/name of the custom version • version_value (string) - string containing custom version • version_status (Svc.VersionStatus) - status of the custom version | Data Structure for custom version Tlm |
| Fw.Enabled | Enum | Các giá trị liệt kê (Enum): • DISABLED: 0 - Disabled state • ENABLED: 1 - Enabled state | Enabled and disabled states |
| FwTimeBaseStoreType | Alias | Kiểu gốc: U16 | The type used to serialize a time base value |
| FwEventIdType | Alias | Kiểu gốc: U32 | The type of an event identifier |

# 2. Lệnh điều khiển (Commands) - 18 mục

| Tên Lệnh | Opcode | Tham số (Arguments) | Mô tả |
| --- | --- | --- | --- |
| ReferenceDeployment.timeHandler.SetCurrentTime | 12288 | • seconds_now (U32) |  |
| CdhCore.cmdDisp.CMD_NO_OP | 16777216 | (Không có) | No-op command |
| CdhCore.cmdDisp.CMD_NO_OP_STRING | 16777217 | • arg1 (string) - The String command argument | No-op string command |
| CdhCore.cmdDisp.CMD_TEST_CMD_1 | 16777218 | • arg1 (I32) - The I32 command argument • arg2 (F32) - The F32 command argument • arg3 (U8) - The U8 command argument | No-op command |
| CdhCore.cmdDisp.CMD_CLEAR_TRACKING | 16777219 | (Không có) | Clear command tracking info to recover from components not returning status |
| CdhCore.events.SET_EVENT_FILTER | 16781312 | • filterLevel (Svc.EventManager.FilterSeverity) - Filter level • filterEnabled (Svc.EventManager.Enabled) - Filter state | Set filter for reporting events. Events are not stored in component. |
| CdhCore.events.SET_ID_FILTER | 16781314 | • ID (FwEventIdType) • idFilterEnabled (Svc.EventManager.Enabled) - ID filter state | Filter a particular ID |
| CdhCore.events.DUMP_FILTER_STATE | 16781315 | (Không có) | Dump the filter states via events |
| CdhCore.health.HLTH_ENABLE | 16785408 | • enable (Fw.Enabled) - whether or not health checks are enabled | A command to enable or disable health checks |
| CdhCore.health.HLTH_PING_ENABLE | 16785409 | • entry (string) - The entry to enable/disable • enable (Fw.Enabled) - whether or not a port is pinged | Ignore a particular ping entry |
| CdhCore.health.HLTH_CHNG_PING | 16785410 | • entry (string) - The entry to modify • warningValue (U32) - Ping warning threshold • fatalValue (U32) - Ping fatal threshold | Change ping value |
| CdhCore.version.ENABLE | 16789504 | • enable (Svc.VersionEnabled) - whether or not Version telemetry is enabled | A command to enable or disable Event verbosity and Telemetry |
| CdhCore.version.VERSION | 16789505 | • version_type (Svc.VersionType) - which version type Event is requested | Report version as Event |
| CdhCore.tlmSend.SET_LEVEL | 16801792 | • level (FwChanIdType) - The I32 command argument | Set telemetry send level |
| CdhCore.tlmSend.SEND_PKT | 16801793 | • id (U32) - The packet ID | Force a packet to be sent |
| ReferenceDeployment.dataHandling.CloseFile | 268632064 | (Không có) | Forces a close of the currently opened file. |
| ReferenceDeployment.dataHandling.DownlinkLatestFiles | 268632065 | • count (U8) - Number of latest files to downlink | Downlinks the latest files. |
| ReferenceDeployment.dataHandling.DownlinkFilesByIndex | 268632066 | • startIndex (U16) - First file index to downlink • endIndex (U16) - Last file index to downlink | Downlinks files from start index to end index in descending order. |

# 3. Sự kiện / Log (Events) - 76 mục

| Tên Sự Kiện | ID | Mức độ | Tham số | Mô tả |
| --- | --- | --- | --- | --- |
| ReferenceDeployment.timeHandler.SetCurrentTimeError | 12288 | WARNING_HI | • status (U32) - The error status | An error occurred while attempting to set the current time |
| CdhCore.cmdDisp.OpCodeRegistered | 16777216 | DIAGNOSTIC | • Opcode (FwOpcodeType) - The opcode to register • port (I32) - The registration port • slot (I32) - The dispatch slot it was placed in |  |
| CdhCore.cmdDisp.OpCodeDispatched | 16777217 | COMMAND | • Opcode (FwOpcodeType) - The opcode dispatched • port (I32) - The port dispatched to | Op code dispatched event |
| CdhCore.cmdDisp.OpCodeCompleted | 16777218 | COMMAND | • Opcode (FwOpcodeType) - The I32 command argument | Op code completed event |
| CdhCore.cmdDisp.OpCodeError | 16777219 | COMMAND | • Opcode (FwOpcodeType) - The opcode with the error • error (Fw.CmdResponse) - The error value | Op code completed with error event |
| CdhCore.cmdDisp.MalformedCommand | 16777220 | WARNING_HI | • Status (Fw.DeserialStatus) - The deserialization error | Received a malformed command packet |
| CdhCore.cmdDisp.InvalidCommand | 16777221 | WARNING_HI | • Opcode (FwOpcodeType) - Invalid opcode | Received an invalid opcode |
| CdhCore.cmdDisp.TooManyCommands | 16777222 | WARNING_HI | • Opcode (FwOpcodeType) - The opcode that overflowed the list | Exceeded the number of commands that can be simultaneously executed |
| CdhCore.cmdDisp.NoOpReceived | 16777223 | ACTIVITY_HI | (Không có) | The command dispatcher has successfully received a NO-OP command |
| CdhCore.cmdDisp.NoOpStringReceived | 16777224 | ACTIVITY_HI | • message (string) - The NO-OP string that is generated | The command dispatcher has successfully received a NO-OP command from GUI with a string |
| CdhCore.cmdDisp.TestCmd1Args | 16777225 | ACTIVITY_HI | • arg1 (I32) - Arg1 • arg2 (F32) - Arg2 • arg3 (U8) - Arg3 | This log event message returns the TEST_CMD_1 arguments. |
| CdhCore.cmdDisp.OpCodeReregistered | 16777226 | DIAGNOSTIC | • Opcode (FwOpcodeType) - The opcode reregistered • port (I32) - The reregistration port | Op code reregistered event |
| CdhCore.cmdDisp.CommandDroppedQueueOverflow | 16777227 | WARNING_HI | • OpCode (FwOpcodeType) - The command opcode dropped • Context (U32) - The call order | This log event reports the Command Sequence Buffer port queue has overflowed. |
| CdhCore.events.SEVERITY_FILTER_STATE | 16781312 | ACTIVITY_LO | • severity (Svc.EventManager.FilterSeverity) - The severity level • enabled (bool) | Dump severity filter state |
| CdhCore.events.ID_FILTER_ENABLED | 16781313 | ACTIVITY_HI | • ID (FwEventIdType) - The ID filtered | Indicate ID is filtered |
| CdhCore.events.ID_FILTER_LIST_FULL | 16781314 | WARNING_LO | • ID (FwEventIdType) - The ID filtered | Attempted to add ID to full ID filter ID |
| CdhCore.events.ID_FILTER_REMOVED | 16781315 | ACTIVITY_HI | • ID (FwEventIdType) - The ID removed | Removed an ID from the filter |
| CdhCore.events.ID_FILTER_NOT_FOUND | 16781316 | WARNING_LO | • ID (FwEventIdType) - The ID removed | ID not in filter |
| CdhCore.health.HLTH_PING_WARN | 16785408 | WARNING_HI | • entry (string) - The entry passing the warning level | Warn that a ping target is longer than the warning value |
| CdhCore.health.HLTH_PING_LATE | 16785409 | FATAL | • entry (string) - The entry passing the warning level | Declare FATAL since task is no longer responding |
| CdhCore.health.HLTH_PING_WRONG_KEY | 16785410 | FATAL | • entry (string) - The entry passing the warning level • badKey (U32) - The incorrect key value | Declare FATAL since task is no longer responding |
| CdhCore.health.HLTH_CHECK_ENABLE | 16785411 | ACTIVITY_HI | • enabled (Fw.Enabled) - If health checking is enabled | Report checking turned on or off |
| CdhCore.health.HLTH_CHECK_PING | 16785412 | ACTIVITY_HI | • enabled (Fw.Enabled) - If health pinging is enabled for a particular entry • entry (string) - The entry passing the warning level | Report a particular entry on or off |
| CdhCore.health.HLTH_CHECK_LOOKUP_ERROR | 16785413 | WARNING_LO | • entry (string) - The entry passing the warning level | Entry was not found |
| CdhCore.health.HLTH_PING_UPDATED | 16785414 | ACTIVITY_HI | • entry (string) - The entry changed • warn (U32) - The new warning value • fatal (U32) - The new FATAL value | Report changed ping |
| CdhCore.health.HLTH_PING_INVALID_VALUES | 16785415 | WARNING_HI | • entry (string) - The entry changed • warn (U32) - The new warning value • fatal (U32) - The new FATAL value | Report changed ping |
| CdhCore.version.FrameworkVersion | 16789504 | ACTIVITY_LO | • version (string) - version string | Version of the git repository. |
| CdhCore.version.ProjectVersion | 16789505 | ACTIVITY_LO | • version (string) - version string | Version of the git repository. |
| CdhCore.version.LibraryVersions | 16789506 | ACTIVITY_LO | • version (string) - version string | Version of the git repository. |
| CdhCore.version.CustomVersions | 16789507 | ACTIVITY_LO | • version_enum (Svc.VersionCfg.VersionEnum) - The enum to access • version_value (string) - version | Version of the git repository. |
| CdhCore.fatalAdapter.AF_ASSERT_0 | 16797696 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_1 | 16797697 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_2 | 16797698 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument • arg2 (U32) - Second assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_3 | 16797699 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument • arg2 (U32) - Second assert argument • arg3 (U32) - Third assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_4 | 16797700 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument • arg2 (U32) - Second assert argument • arg3 (U32) - Third assert argument • arg4 (U32) - Fourth assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_5 | 16797701 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument • arg2 (U32) - Second assert argument • arg3 (U32) - Third assert argument • arg4 (U32) - Fourth assert argument • arg5 (U32) - Fifth assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_ASSERT_6 | 16797702 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • arg1 (U32) - First assert argument • arg2 (U32) - Second assert argument • arg3 (U32) - Third assert argument • arg4 (U32) - Fourth assert argument • arg5 (U32) - Fifth assert argument • arg6 (U32) - Sixth assert argument | An assert happened |
| CdhCore.fatalAdapter.AF_UNEXPECTED_ASSERT | 16797703 | FATAL | • file (string) - The source file of the assert • line (U32) - Line number of the assert • numArgs (U32) - Number of unexpected arguments | An unexpected assert happened |
| CdhCore.tlmSend.NoChan | 16801792 | WARNING_LO | • Id (FwChanIdType) - The telemetry ID | Telemetry channel is not part of a telemetry packet. |
| CdhCore.tlmSend.LevelSet | 16801793 | ACTIVITY_HI | • id (FwChanIdType) - The level | Telemetry send level set |
| CdhCore.tlmSend.MaxLevelExceed | 16801794 | WARNING_LO | • level (FwChanIdType) - The level • max (FwChanIdType) - The max packet level | Telemetry send level set |
| CdhCore.tlmSend.PacketSent | 16801795 | ACTIVITY_LO | • id (U32) - The packet ID | Packet manually sent |
| CdhCore.tlmSend.PacketNotFound | 16801796 | WARNING_LO | • id (U32) - The packet ID | Couldn't find the packet to send |
| ComCcsds.comQueue.QueueOverflow | 33554432 | WARNING_HI | • queueType (Svc.QueueType) - The Queue data type • index (U32) - index of overflowed queue | Queue overflow event |
| ComCcsds.frameAccumulator.NoBufferAvailable | 33558528 | WARNING_HI | (Không có) | An error occurred while deserializing a packet |
| ComCcsds.frameAccumulator.FrameDetectionSizeError | 33558529 | WARNING_HI | • size_out (FwSizeType) | A frame was detected whose size exceeds the internal accumulation buffer capacity. No choice but to drop the frame. |
| ComCcsds.commsBufferManager.NoBuffsAvailable | 33562624 | WARNING_HI | • size (FwSizeType) - The requested size | The BufferManager was unable to allocate a requested buffer |
| ComCcsds.commsBufferManager.NullEmptyBuffer | 33562625 | WARNING_HI | (Không có) | The buffer manager received a null pointer and zero-sized buffer as a return. Probably undetected failed buffer allocation |
| ComCcsds.fprimeRouter.SerializationError | 33566720 | WARNING_HI | • status (U32) - The status of the operation | An error occurred while serializing a com buffer |
| ComCcsds.fprimeRouter.DeserializationError | 33566721 | WARNING_HI | • status (U32) - The status of the operation | An error occurred while deserializing a packet |
| ComCcsds.fprimeRouter.AllocationError | 33566722 | WARNING_HI | • reason (Svc.FprimeRouter.AllocationReason) | An allocation error occurred |
| ComCcsds.tcDeframer.InvalidSpacecraftId | 33570816 | WARNING_LO | • transmitted (U16) • configured (U16) | Deframing received an invalid SCID |
| ComCcsds.tcDeframer.InvalidFrameLength | 33570817 | WARNING_HI | • transmitted (U16) • actual (FwSizeType) | Deframing received an invalid frame length |
| ComCcsds.tcDeframer.InvalidVcId | 33570818 | ACTIVITY_LO | • transmitted (U16) • configured (U16) | Deframing received an invalid VCID |
| ComCcsds.tcDeframer.InvalidCrc | 33570819 | WARNING_HI | • transmitted (U16) • computed (U16) | Deframing received an invalid checksum |
| ComCcsds.spacePacketDeframer.InvalidLength | 33574912 | WARNING_HI | • transmitted (U16) • actual (FwSizeType) | Deframing received an invalid frame length |
| ComCcsds.apidManager.UnexpectedSequenceCount | 33591296 | WARNING_LO | • transmitted (U16) • expected (U16) | Deframing received an unexpected sequence count |
| ComCcsds.apidManager.ApidTableFull | 33591297 | WARNING_HI | • invalidApidValue (U16) | Received an unregistered APID |
| ReferenceDeployment.framDriver.FrStartOk | 268566528 | DIAGNOSTIC | (Không có) |  |
| ReferenceDeployment.framDriver.FrStartFailed | 268566529 | WARNING_HI | • status (ISIS.FramStatus) |  |
| ReferenceDeployment.framDriver.FrWriteFailed | 268566530 | WARNING_HI | • address (U32) • length (U32) • status (ISIS.FramStatus) |  |
| ReferenceDeployment.framDriver.FrReadFailed | 268566531 | WARNING_HI | • address (U32) • length (U32) • status (ISIS.FramStatus) |  |
| ReferenceDeployment.framDriver.FrWriteVerifyFailed | 268566532 | WARNING_HI | • address (U32) • length (U32) • status (ISIS.FramStatus) |  |
| ReferenceDeployment.dataHandling.FileOpenError | 268632064 | WARNING_HI | • errornum (U32) - The error number returned from open file • file (string) - The file | The ComLogger encountered an error opening a file |
| ReferenceDeployment.dataHandling.FileWriteError | 268632065 | WARNING_HI | • errornum (U32) - The error number returned from write file • bytesWritten (U32) - The number of bytes successfully written to file • bytesToWrite (U32) - The number of bytes attempted to write to file • file (string) - The file | The ComLogger encountered an error writing to a file |
| ReferenceDeployment.dataHandling.FileValidationError | 268632066 | WARNING_LO | • validationFile (string) - The validation file • file (string) - The file • status (U32) - The Os::Validate::Status return | The ComLogger encountered an error writing the validation file |
| ReferenceDeployment.dataHandling.FileClosed | 268632067 | DIAGNOSTIC | • file (string) - The file | The ComLogger successfully closed a file on command. |
| ReferenceDeployment.dataHandling.FileNotInitialized | 268632068 | WARNING_LO | (Không có) |  |
| ReferenceDeployment.dataHandling.PrmIdNotFound | 268632080 | WARNING_LO | • Id (FwPrmIdType) - The parameter ID | Parameter ID not found in database. |
| ReferenceDeployment.dataHandling.PrmIdUpdated | 268632081 | ACTIVITY_HI | • Id (FwPrmIdType) - The parameter ID | Parameter ID updated in database |
| ReferenceDeployment.dataHandling.PrmDbFull | 268632082 | FATAL | • Id (FwPrmIdType) - The parameter ID | Parameter database is full |
| ReferenceDeployment.dataHandling.PrmIdAdded | 268632083 | ACTIVITY_HI | • Id (FwPrmIdType) - The parameter ID | Parameter ID added to database |
| ReferenceDeployment.dataHandling.PrmWriteError | 268632084 | WARNING_HI | • stage (Vspc.DataHandling.PrmWriteError) - The write stage • record (I32) - The record that had the failure • error (I32) - The error code | Failed to write parameter data |
| ReferenceDeployment.dataHandling.PrmSaveComplete | 268632085 | ACTIVITY_HI | • records (U32) - The number of records saved | Save of parameter data completed |
| ReferenceDeployment.dataHandling.PrmReadError | 268632086 | WARNING_HI | • stage (Vspc.DataHandling.PrmReadError) - The read stage • record (I32) - The record that had the failure • error (I32) - The error code | Failed to read parameter data |
| ReferenceDeployment.dataHandling.PrmLoadComplete | 268632087 | ACTIVITY_HI | • records (U32) - The number of records loaded | Load of parameter data completed |

# 4. Kênh dữ liệu từ xa (Telemetry) - 43 mục

| Tên Kênh | ID | Kiểu dữ liệu | Mô tả |
| --- | --- | --- | --- |
| ReferenceDeployment.rateGroup100Hz.MaxCycleTime | 4096 | U32 | Max execution time of rate group cycle |
| ReferenceDeployment.rateGroup100Hz.CycleTime | 4097 | U32 | Execution time of current cycle |
| ReferenceDeployment.rateGroup100Hz.CycleCount | 4098 | U32 | Count of number of cycles |
| ReferenceDeployment.rateGroup10Hz.MaxCycleTime | 4352 | U32 | Max execution time of rate group cycle |
| ReferenceDeployment.rateGroup10Hz.CycleTime | 4353 | U32 | Execution time of current cycle |
| ReferenceDeployment.rateGroup10Hz.CycleCount | 4354 | U32 | Count of number of cycles |
| ReferenceDeployment.rateGroup5Hz.MaxCycleTime | 4608 | U32 | Max execution time of rate group cycle |
| ReferenceDeployment.rateGroup5Hz.CycleTime | 4609 | U32 | Execution time of current cycle |
| ReferenceDeployment.rateGroup5Hz.CycleCount | 4610 | U32 | Count of number of cycles |
| CdhCore.cmdDisp.CommandsDispatched | 16777216 | U32 | Number of commands dispatched |
| CdhCore.cmdDisp.CommandErrors | 16777217 | U32 | Number of command errors |
| CdhCore.cmdDisp.CommandsDropped | 16777218 | U32 | Number of commands drooped due to buffer overflow |
| CdhCore.health.PingLateWarnings | 16785408 | U32 | Number of overrun warnings |
| CdhCore.version.FrameworkVersion | 16789504 | string | Software framework version |
| CdhCore.version.ProjectVersion | 16789505 | string | Software project version |
| CdhCore.version.CustomVersion01 | 16789506 | Svc.CustomVersionDb | Custom Versions |
| CdhCore.version.CustomVersion02 | 16789507 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion03 | 16789508 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion04 | 16789509 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion05 | 16789510 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion06 | 16789511 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion07 | 16789512 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion08 | 16789513 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion09 | 16789514 | Svc.CustomVersionDb |  |
| CdhCore.version.CustomVersion10 | 16789515 | Svc.CustomVersionDb |  |
| CdhCore.version.LibraryVersion01 | 16789516 | string | Library Versions |
| CdhCore.version.LibraryVersion02 | 16789517 | string |  |
| CdhCore.version.LibraryVersion03 | 16789518 | string |  |
| CdhCore.version.LibraryVersion04 | 16789519 | string |  |
| CdhCore.version.LibraryVersion05 | 16789520 | string |  |
| CdhCore.version.LibraryVersion06 | 16789521 | string |  |
| CdhCore.version.LibraryVersion07 | 16789522 | string |  |
| CdhCore.version.LibraryVersion08 | 16789523 | string |  |
| CdhCore.version.LibraryVersion09 | 16789524 | string |  |
| CdhCore.version.LibraryVersion10 | 16789525 | string |  |
| CdhCore.tlmSend.SendLevel | 16801792 | FwChanIdType | Telemetry send level |
| ComCcsds.comQueue.comQueueDepth | 33554432 | Svc.ComQueueDepth | Depth of queues of Fw::ComBuffer type |
| ComCcsds.comQueue.buffQueueDepth | 33554433 | Svc.BuffQueueDepth | Depth of queues of Fw::Buffer type |
| ComCcsds.commsBufferManager.TotalBuffs | 33562624 | U32 | The total buffers allocated |
| ComCcsds.commsBufferManager.CurrBuffs | 33562625 | U32 | The current number of allocated buffers |
| ComCcsds.commsBufferManager.HiBuffs | 33562626 | U32 | The high water mark of allocated buffers |
| ComCcsds.commsBufferManager.NoBuffs | 33562627 | U32 | The number of requests that couldn't return a buffer |
| ComCcsds.commsBufferManager.EmptyBuffs | 33562628 | U32 | The number of empty buffers returned |